Lena filtered water sample.

Method – Three different functionalised grids where used. Positively charged (attracts negatively charged particles), negatively charged (attracted positively charge particles) and PEG coated (good for organics).  Fe-rich particles where found on both –ve and +ve grids, but –ve grids collected more Fe-rich particles of interest.  PEG coated grids where not that effective for Fe-rich particles. The grids where held in tweezers and stirred in the 0.22um filtered water sample for ~3 mins, then washed in IPA and air dried prior to investigation in the TEM. Images and EDX spectra where collected. SAED of particles was attempted but no usable patterns collected due to interferences with the grid coating material. Sample 51, 55 and 66 were investigated. C and Cu in EDX spectra are due to the grids. Grids also contain small amounts of Si.

Results.

Original data files are available at

http://homepages.see.leeds.ac.uk/~earitb/LenaTEM/

Interpretation - Based on morphology and composition data below it is very likely that the Fe fraction (present in Lena River water samples) that passes 0.22um filters but retained on 10kDa filters is 20-30 nm aggregates of 2-5 nm ferrihydrate particles. These were the only Fe-containing discovered.

Results – Sample 51 +ve grid; particle 0002

Typical of the FHY particles present.

Results – Sample 51 +ve grid; particle 0006

More FHY

Results – Sample 51 +ve grid; particle 0010

A rare Al-Si-O rich particle (mica or clay?)

Results – Sample 51 PEG grid; particle 0001

Ca-rich (Ca carbonate?)

Results – Sample 51 PEG grid; particle 0003

Organic particle

Results – Sample 51 –ve grid; particle 0001

FHY? – Ca /Si rich too – some FHY had this composition.

Results – Sample 51 –ve grid; particle 0006

More FHY – very typical example.

Results – Sample 55 –ve grid; particle 0001

Results – Sample 55 –ve grid; particle 0004

Results – Sample 66 –ve grid; particle 0001